# Router Configuration and Routing Protocols using Cisco Packet Tracer

## Computer Networks Practical Assignment

### Submitted By

Student Name: MAND Fernando

Student ID: 39271

Module Name: Computer Networks

Assignment Title: Router Configuration and Routing Protocols using Cisco Packet Tracer

Submission Date: 15 June 2026

# Introduction

This assignment demonstrates the configuration of Cisco routers using Cisco Packet Tracer. The practical activities include basic router configuration, implementation of static routing, and dynamic routing using RIP and EIGRP protocols. The objective is to establish successful communication among multiple networks and verify connectivity through routing tables and ping tests.

# Task 1 – Basic Router Configuration

## Objective

Configure the router hostname, secure console access with a password, and enable login authentication.

### Configuration Commands

enable
configure terminal

hostname KandyNSBM_39271

line console 0
password NSBM
login

end
write memory

### Explanation of Commands

| Command | Purpose |
| --- | --- |
| hostname KandyNSBM_39271 | Changes the router hostname |
| line console 0 | Accesses console configuration mode |
| password NSBM | Sets the console password |
| login | Enables password authentication |
| write memory | Saves configuration permanently |

Hostname configuration and Console password configuration

### Result

The router hostname was successfully changed to KandyNSBM_39271 and console access was secured using the password NSBM.

# Task 2 – Static Routing Configuration

## Network Topology

The network consists of three routers:

- ComputingRouter

- BusinessRouter

- ScienceRouter

### Network Addressing

| Device | Interface | IP Address |
| --- | --- | --- |
| ComputingRouter | G0/2 | 192.168.1.1 |
| ComputingRouter | G0/1 | 12.0.0.1 |
| ComputingRouter | G0/0 | 11.0.0.1 |
| BusinessRouter | G0/2 | 192.168.2.1 |
| BusinessRouter | G0/1 | 12.0.0.2 |
| BusinessRouter | G0/0 | 10.0.0.1 |
| ScienceRouter | G0/1 | 192.168.3.1 |
| ScienceRouter | G0/0 | 10.0.0.2 |
| ScienceRouter | G0/2 | 11.0.0.2 |

## Router Configurations

### ComputingRouter

interface g0/2
ip address 192.168.1.1 255.255.255.0
no shutdown

interface g0/1
ip address 12.0.0.1 255.255.255.252
no shutdown

interface g0/0
ip address 11.0.0.1 255.255.255.252
no shutdown

ip route 192.168.2.0 255.255.255.0 12.0.0.2
ip route 192.168.3.0 255.255.255.0 11.0.0.2

### BusinessRouter

interface g0/2
ip address 192.168.2.1 255.255.255.0
no shutdown

interface g0/0
ip address 10.0.0.1 255.255.255.252
no shutdown

interface g0/1
ip address 12.0.0.2 255.255.255.252
no shutdown

ip route 192.168.1.0 255.255.255.0 12.0.0.1
ip route 192.168.3.0 255.255.255.0 10.0.0.2

### ScienceRouter

interface g0/1
ip address 192.168.3.1 255.255.255.0
no shutdown

interface g0/0
ip address 10.0.0.2 255.255.255.252
no shutdown

interface g0/2
ip address 11.0.0.2 255.255.255.252
no shutdown

ip route 192.168.1.0 255.255.255.0 11.0.0.1
ip route 192.168.2.0 255.255.255.0 10.0.0.1

## Explanation of Static Routing

Static routing manually defines routes to remote networks. Each router is configured with destination networks and the next-hop address to reach those networks. This allows routers to forward packets correctly between different LANs.

Network topology diagram

ComputingRouter configuration

BusinessRouter configuration

ScienceRouter configuration

ComputingRouter routing table

BusinessRouter routing table

ScienceRouter routing table

Ping Test Network A ↔ Network B

Ping Test Network A ↔ Network C

Ping Test Network B ↔ Network C

### Results

All routers successfully exchanged packets using static routes. Ping tests confirmed successful communication between all networks.

# Task 3 – Dynamic Routing Configuration

## Part A – RIP Routing

### Objective

Configure Routing Information Protocol (RIP Version 2) to automatically exchange routing information among routers.

## RIP Configuration

### KandyNSBM Router

router rip
version 2
no auto-summary
network 14.0.0.0
network 172.16.0.0

### ColomboNSBM Router

router rip
version 2
no auto-summary
network 14.0.0.0
network 15.0.0.0

### GalleNSBM Router

router rip
version 2
no auto-summary
network 15.0.0.0
network 173.16.0.0

### Explanation

RIP is a distance-vector routing protocol that exchanges routing information periodically. Version 2 supports subnet masks and classless routing.

RIP Network Diagram

KandyNSBM RIP Configuration

ColomboNSBM RIP Configuration

GalleNSBM RIP Configuration

Successful Ping Test

### Result

RIP successfully learned remote routes and enabled communication between all networks.

# Part B – EIGRP Routing

## Objective

Replace RIP with EIGRP and verify network connectivity.

### KandyNSBM Router

no router rip

router eigrp 100
no auto-summary
network 14.0.0.0 0.0.0.3
network 172.16.0.0 0.0.255.255

### ColomboNSBM Router

no router rip

router eigrp 100
no auto-summary
network 14.0.0.0 0.0.0.3
network 15.0.0.0 0.0.0.3

### GalleNSBM Router

no router rip

router eigrp 100
no auto-summary
network 15.0.0.0 0.0.0.3
network 173.16.0.0 0.0.255.255

### Explanation

EIGRP is an advanced distance-vector routing protocol developed by Cisco. It converges faster than RIP and uses metrics based on bandwidth and delay to determine optimal routes.

KandyNSBM EIGRP Configuration

ColomboNSBM EIGRP Configuration

GalleNSBM EIGRP Configuration

KandyNSBM Routing Table

KandyNSBM EIGRP Neighbors

ColomboNSBM Routing Table

ColomboNSBM EIGRP Neighbors

GalleNSBM Routing Table

GalleNSBM EIGRP Neighbors

Successful Ping Test

### Result

EIGRP established neighbor relationships successfully and dynamically exchanged routes among all routers. Connectivity tests confirmed successful communication between all networks.

# Conclusion

This practical assignment provided hands-on experience with Cisco router configuration and routing technologies. Basic router security was implemented through hostname configuration and console authentication. Static routing was configured manually to establish communication among three separate networks. Dynamic routing was then implemented using RIP and EIGRP protocols. Connectivity was verified through routing tables and successful ping tests. The assignment demonstrated the differences between static and dynamic routing and highlighted the efficiency of EIGRP in large-scale network environments.

# GitHub Repository

Repository Name: CNRoutingAssignment

Repository Contents:

- Task 1 Packet Tracer File

- Task 2 Packet Tracer File

- Task 3 Packet Tracer File

- README.md Description

- Assignment Report PDF

GitHub Link: https://github.com/yourusername/CNRoutingAssignment